LECTURE 1

- Definition

Combinational devices: their outputs depend only on the current input combination (i.e., the combination of their input values)

- Definition

Sequential devices: their outputs depend on the current input combination and the sequence of past inputs.

LECTURE 2

- Axioms of Switching Algebra

| A1. X=0 if X≠1 A2. if X=0, then X'=1 A3. 0∙0=0 A5. 1∙1=1 A6. 0∙1=1∙0=0 | A1'. X=1 if X≠0 A2'. if X=1, then X'=0 A3'. 0+0=0 A4'. 1+1=1 A5'. 0+1=1+0=1 |
| --- | --- |

- Theorems of Switching Algebra

| T1. X+0=X T2. X+1=1 T3. X+X=X T4. (X')'=X T5. X+X'=1 | T1'. X∙1=X T2'. X∙0=0 T3'. X∙X=X T5'. X∙X'=0 | Identity Null Element Idempotency Involution Complement |
| --- | --- | --- |
| T6. X+Y=Y+X T7. X+Y+Z=X+(Y+Z) T8. X∙Y+X∙Z=X∙(Y+Z) T9. X+X∙Y=X T10. X∙Y+X∙Y'=X T11. X∙Y+X'∙Z+Y∙Z=X∙Y+X'∙Z | T6'. X∙Y=Y∙X T7'. X∙Y∙Z=X∙(Y∙Z) T8'. X+Y∙X+Z=X+Y∙Z T9'. X∙(X+Y)=X T10'. (X+Y)∙(X+Y')=X T11'.X+Y∙X'+Z∙Y+Z=X+Y∙X'+Z | Commutativity Associativity Distributivity Covering Combining Consensus |

| T12. X+X+…+X=X T12'. X∙X∙…∙X=X | Generalized Idempotency |
| --- | --- |
| T13. X1∙X2∙…∙Xn'=X1'+X2'+…+Xn' T13'. X1+X2+…+Xn'=X1'∙X2'∙…∙Xn' | DeMorgan's Theorems |
| T14. [FX1,X2,…,Xn,+,∙]'=F(X1',X2',…,Xn',∙,+) | Generalized DeMorgan's |
| T15. FX1,X2,…,Xn=X1∙F1,X2,…,Xn+X1'∙F0,X2,…,Xn T15'. FX1,X2,…,Xn=[X1+F0,X2,…,Xn]∙[X1'+F1,X2,…,Xn] | Shannon's Expansion |

- Duality

FDX1,X2,…,Xn,+,∙,'=FX1,X2,…,Xn,∙,+,'

- Minterms & Maxterms

A,B,C(0, 1, 2, 3)= A,B,C(4, 5, 6, 7)

LECTURE 3

- Sum-of-Products -> Product-of-Sums

a·b+c·d=a+c·a+d·b+c·b+d

- AND-OR -> NAND-NAND

- OR-AND -> NOR-NOR

LECTURE 4-5

- Karnaugh Maps

- Minimizing Sum-of-Product

- “Don’t Care” Inputs (d)

- distinguished 1-cells

- prime implicant (essential & non-essential)

- complete sum & minimal sum

LECTURE 6-7

- Multiplexer – 2n-to-1 (n inputs)

- Decoder – n-to-2n

- Encoder – 2n-to-n

  - Expanding MUXs & Decoders

  - Implementing Functions w/ Combinational Circuits

- Read-Only Memory (ROM) – n inputs -> 2n options, m outputs

- Programmable Logic Arrays (PLA)

- Tri-State Buffer

LECTURE 8-9

- S-R Latch (Set-Reset Latch)

- D Latch

- Edge-Triggered D flip-flop

- S-R flip-flop

- J-K flip-flop

- T flip-flop

LECTURE 10

- 1s Counting Machine

LECTURE 11-12

LECTURE 13