BIPOLAR JUNCTION TRANSISTOR

A Bipolar Junction Transistor (BJT) is a semiconductor device formed by two pn

junctions. Therefore, it will have three alternating regions. It can be either a narrow n-type region placed between two p type layers (forming a pnp transistor), or a thin p-type region between two n-type layers (representing a npn transistor).

BJT OUTPUT CHARACTERISTIC

- Theoretical Summary

The output characteristic relates the output current iC with the output voltage vCE,

with the condition that the input current iB is kept constant:

iC=iCvCE | iB=const.



- Components

- EB , EC – DC Voltage Sources

- npn Transistor

- RB – Base Resistor

- RC – Collector Resistor

- Values

- RB = 100kΩ

- RC = 1kΩ

- EC ∈[0V, 12V]

- I. EB = 1.6V => (KVL – left loop) => IB1 = 10mA

- II. EB = 2.6V => … => IB2 = 20mA

- Measurements

IB1 = 10 mA                                                      IB2 = 20mA

| EC[V] | UCE[V] | IC[mA] | EC[V] | UCE[V] | IC[mA] |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1.13 | 0.1 | 1.03 | 1.13 | 0.07 | 1.06 |
| 1.37 | 0.11 | 1.26 | 1.29 | 0.08 | 1.21 |
| 1.92 | 0.12 | 1.8 | 1.34 | 0.08 | 1.26 |
| 2.2 | 0.13 | 2.07 | 1.5 | 0.08 | 1.42 |
| 2.6 | 0.15 | 2.45 | 1.73 | 0.09 | 1.64 |
| 2.88 | 0.16 | 2.72 | 2.25 | 0.1 | 2.15 |
| 3.11 | 0.18 | 2.93 | 2.53 | 0.11 | 2.42 |
| 3.39 | 0.2 | 3.19 | 2.8 | 0.12 | 2.68 |
| 3.61 | 0.24 | 3.37 | 3.18 | 0.13 | 3.05 |
| 3.72 | 0.28 | 3.44 | 3.53 | 0.13 | 3.4 |
| 4.03 | 0.5 | 3.53 | 4.17 | 0.15 | 4.02 |
| 4.35 | 0.79 | 3.56 | 4.73 | 0.16 | 4.57 |
| 4.84 | 1.22 | 3.62 | 5.12 | 0.17 | 4.95 |
| 5.18 | 1.53 | 3.65 | 5.78 | 0.19 | 5.59 |
| 6 | 2.28 | 3.72 | 6.27 | 0.22 | 6.05 |
| 7.32 | 3.5 | 3.82 | 6.6 | 0.24 | 6.36 |
| 8.4 | 4.5 | 3.9 | 7 | 0.27 | 6.73 |
| 8.97 | 5.02 | 3.95 | 7.5 | 0.37 | 7.13 |
| 10.33 | 6.3 | 4.03 | 8.15 | 0.78 | 7.37 |
| 10.78 | 6.7 | 4.08 | 8.94 | 1.4 | 7.54 |
| 11.35 | 7.23 | 4.12 | 9.47 | 1.82 | 7.65 |
| 12.2 | 8.06 | 4.14 | 9.89 | 2.16 | 7.73 |
|  |  |  | 10.49 | 2.64 | 7.85 |
|  |  |  | 10.75 | 2.85 | 7.9 |
|  |  |  | 11.21 | 3.22 | 7.99 |
|  |  |  | 11.57 | 3.5 | 8.07 |
|  |  |  | 12.04 | 3.9 | 8.14 |
|  |  |  | 12.2 | 4.02 | 8.18 |

- KVL (right loop):

EC=RC∙IC + UCE ⇒ IC= EC - UCERC

- Current Gain β

- IB = 10mA

β= ICIBn=0.306

- IB = 20mA

β= ICIBn=0.248

- Experimental Model

- Simulations

- Conclusion

The collector current IC, and in turn the Q point’s current ICQ and voltage VCEQ, depends on the value of the base current IB.

IC is directly proportional with IB.

BJT VOLTAGE DIVIDER (SELF-BIAS)

- Theoretical Summary

The operating point (Q point or bias point) of the BJT is defined as the DC

component pair of the collector current ICQ and the collector – emitter voltage VCEQ.

Graphically, the operating point is situated at the intersection of the load line and

the output I-V characteristic of the transistor.

- Components

- EC – DC Voltage Sources

- npn Transistor

- RB1 , RB2 – Base Resistors

- RC – Collector Resistor

- RE – Emitter Resistor

- Values

- EC = 12V

- RC = 2.2kΩ

- RE = 1kΩ

- Depending on the potentiometer:

  - I. P ⟶ M: RB1 = 47kΩ & RB2 = 20kΩ

  - II. P ⟶ N: RB1 = 57kΩ & RB2 = 10kΩ

- Theoretical Analysis & Calculations

  - Thevenin Method

| Linear Circuit Load |  |
| --- | --- |

Q:ICQUCEQ⇒ IE ≅ ICQICQ= β ∙ IBUBE0=0.6v

KVL: EBB= IB∙RBB+ VBE0+ IE∙REEC= ICQ∙RC+ VCEQ+ IE∙RE⇒ ICQ= β∙EBB-VBE0RBB+ β∙REVCEQ= EC- ICQ(RC+RE)

- P ⟶ M :  RB1 = 47kΩ & RB2 = 20kΩ

EBB= RB2RB2+ RB1∙EC=3.58V

RBB= RB1 ∥RB2= 14kΩ

- β=333

ICQ=2.85mA

VCEQ=2.88V

- β=130

ICQ=2.69mA

VCEQ=3.39V

- P ⟶ N :  RB1 = 57kΩ & RB2 = 10kΩ

EBB= RB2RB2+ RB1∙EC=1.79V

RBB= RB1 ∥RB2= 8.5kΩ

- β=333

ICQ=1.16mA

VCEQ=8.28V

- β=130

ICQ=1.11mA

VCEQ= 8.44V

|  | β = 333 |  | β = 130 |  |
| --- | --- | --- | --- | --- |
|  | ICQ [mA] | UCEQ [V] | ICQ [mA] | UCEQ [V] |
| P ⟶ M | 2.85 | 2.88 | 2.69 | 3.39 |
| P ⟶ N | 1.16 | 8.28 | 1.11 | 8.44 |

β = 333

β = 130

- Experimental Model

- Measurements

| θ = 25°C | T1 |  | T2 |  |
| --- | --- | --- | --- | --- |
|  | β = 333 |  | β = 130 |  |
|  | ICQ [mA] | UCEQ [V] | ICQ [mA] | UCEQ [V] |
| P ⟶ M | 2.83 | 2.95 | 2.64 | 3.54 |
| P ⟶ N | 1.15 | 8.32 | 1.08 | 8.51 |

β = 333

β = 130

- Simulations

| P ⟶ M | P ⟶ N |
| --- | --- |
| β = 166 ICQ = 2.67mA UCEQ = 3.44V | β = 163 ICQ = 1.08mA UCEQ = 8.53V |

- Conclusion

The Q point of the self-bias voltage divider circuit depends more on the resistances at the base of the transistor, RB1 and RB2, than the variation of the transistor’s current gain, β.

BJT AMPLIFIER

- Theoretical Summary

The typical voltage amplifier using a single BJT – common emitter connection. It can be observed that this circuit contains two voltage sources. The DC voltage source EC is required for biasing the transistor in the active region so that it can work as an amplifier. On the other hand, the AC source Eg provides the variable signal to be amplified.

- Components

- EC – DC Voltage Source

- Eg – AC Voltage Source

- npn Transistor

- C1 , C2 – Capacitors

- CE – Emitter Capacitor

- RB1 , RB2 – Base Resistors

- RC – Collector Resistor

- RE1 , RE2 – Emitter Resistors

- Rg – Input Internal Resistor (Eg)

- RL – Load Resistor

- Values

| - EC = +5V - C1 = C2 = 4.7µF - RC = 2.2kΩ - RB1 = 22kΩ - RE1 = 100Ω - RL = 10kΩ - βmin = 100 - SWITCH K: K⟶ 1, K⟶ 2, K⟶ 3 | - CE = 470µF - RB2 = 10kΩ - RE2 = 900Ω - Rg = 50Ω - β = h21e = 307/25°C |
| --- | --- |

- Theoretical Analysis & Calculations

- DC Analysis

Q:ICQ=0.95mAUCEQ=1.92V

- AC Analysis

gm=h21eh11e≅40∙ICQ=38∙10-3

RB= RB1∥RB2=6.8kΩ

RCI=RC∥RL=1.8kΩ

IC=h21e∙IB

- K ⟶ 1:

Au=VoutVin=VoutIc∙IcIb∙IbVin=-RCI∙h21e∙1h11e=-gm∙RCI≅-68.4

- K ⟶ 2:

Au=VoutVin=VoutIc∙IcIb∙IbVin=-RCI∙h21e∙IBh11e∙IB+RE1IB+IC

=-RCI∙h21e∙1h11e+RE11+h21e≅-14.21

- K ⟶ 3:

RE=RE1+RE2=1kΩ

Au=VoutVin=VoutIc∙IcIb∙IbVin=-RCI∙h21e∙1h11e+RE1+h21e≅-1.74

- Experimental Model

| K ⟶ 1 | K ⟶ 2 |
| --- | --- |
| K ⟶ 3 |  |

- Measurements

K ⟶ 1

Vp(in)=Vpp(yel)2=28mV Vp(out)=Vpp(pur)2=1.44V Au=-VpoutVpin=-51.43

K ⟶ 2

Vp(in)=Vpp(yel)2=52mV Vp(out)=Vpp(pur)2=695mV Au=-Vp(out)Vp(in)=-13.36

K ⟶ 3

Vp(in)=Vpp(yel)2=500mV Vp(out)=Vpp(pur)2=880mV Au=-Vp(out)Vp(in)=-1.76

- Simulations

K ⟶ 1

Vp(in)=20mV Vp(out)=Vpp(out)2=2.73V2=1.37V Au=-Vp(out)Vp(in)=-68.5

K ⟶ 2

Vp(in)=20mV Vp(out)=Vpp(out)2=546mV2=273mV Au=-VpoutVpin=-13.65

K ⟶ 3

Vp(in)=20mV Vp(out)=Vpp(out)2=70mV2=35mV Au=-Vp(out)Vp(in)=-1.75

- Table

|  | K ⟶ 1 | K ⟶ 2 | K ⟶ 3 |
| --- | --- | --- | --- |
| Calculation | -68.4 | -14.21 | -1.74 |
| Experimental | -51.43 | -13.36 | -1.76 |
| Simulation | -68.5 | -13.65 | -1.75 |

- Conclusion

The BJT can be used as an AC voltage amplifier and depending on the resistance RE before the emitter capacitor we can change the voltage gain from the input voltage to the output voltage, the lower the resistance the higher the absolute voltage gain and the higher the resistance the lower the absolute voltage gain( |Au| i.p. RE ).

As RE→0 the output voltage waveform appears to deform from the input sinusoidal waveform.