THE HALF WAVE RECTIFIER

- Theoretical Summary

A rectifier is a device that converts the sinusoidal alternating current ( AC ) into somewhat constant direct current ( DC ). Half wave rectifiers converts only one half-cycle of the AC voltage ( positive in our case ) and will block the other half.

The half wave rectifier is connected to a sinusoidal AC voltage source of 220V RMS and 50Hz through a step-down transformer, that steps the voltage down to ≈ 13.3V RMS ( ≈ 37.8V peak to peak).

- Components

- Tr – Step-Down Transformer

- used to lower the voltage between the 2 circuits, while also increasing the current by the same factor

- N1 – nr. of coil turns from I

- N2 – nr. of coil turns from II ( < N1 )

- V1m – voltage amplitude of I

- V2m – voltage amplitude of II

- n=N1N2= V1mV2m – transformation ratio

- D – Diode

- constitutes the rectifier of the circuit

- allows current to pass only in one direction, the positive half of the input sinusoidal wave

- C – Capacitor

- makes up the low-pass filter

- used to smoothen the pulsating DC waveform

- RL – Load Resistor

- Values

- RL = 5kΩ

- C – cases:

- No capacitor

- 1 capacitor – C = 22µF

- 2 capacitors in parallel – C = 44µF

- 3 capacitors in parallel – C = 66µF

- Formulas

- Vm = Vpp2

- T= 1f

- VLDC=V2mπ ( no capacitor )

- ΔvL=V2mC∙R∙f , VLDC= V2m- ΔvL2 ( with capacitor )

- Experimental Procedure

- make the circuit above

- using a multimeter ( VAC domain ), it is measured the voltage given by the transformer secondary -> amplitude ( A = V2m ) & time period ( T )

- For the 4 cases: SW open, SW closed(1C, 2C & 3C):

  - output waveform vL(t) is observed

  - measurements are took for the DC component of the output voltage VLDC and the ripple ∆vL

- Measurements – taken from screenshots of oscilloscope(lab 3)

- f = 50Hz =>T=20 ms=0.02s

- Case I – No C

V2pp= 37.8V=>V2m= 37.82=18.9V

VLDC=5.80V

- Case II – 1 C

V2pp= 37.4V=>V2m= 37.42=18.7V

ΔvL=1.88V

VLDC=17.76V

- Case III – 2 C

V2pp= 37.4V=>V2m= 37.42=18.7V

ΔvL=1.1V

VLDC=18.15V

- Case IV – 3 C

V2pp= 37.4V=>V2m= 37.42=18.7V

ΔvL=0.74V

VLDC=18.33V

| CASE I | CASE II |
| --- | --- |
| CASE III | CASE IV |

- Calculations

- V2pp= 37.8V=>V2m= 37.82=18.9V

- f = 50Hz =>T=20 ms=0.02s

- Case I – No C

VLDC=18.9π=6.02V

- Case II – 1 C

ΔvL=18.922∙5 ∙50 ∙ 10-3=3.44V

VLDC= 18.9- 3.442=17.18V

- Case III – 2 C

ΔvL=18.944∙5 ∙50 ∙ 10-3=1.72V

VLDC= 18.9- 1.722=18.04V

- Case IV – 3 C

ΔvL=18.966∙5 ∙50 ∙ 10-3=1.15V

VLDC= 18.9- 1.152=18.33V

- Simulations

CASE I

CASE II

CASE III

CASE IV

- Tables

- Experimental

| CAPACITORS | V2m | ΔvL | VLDC |
| --- | --- | --- | --- |
| 0 | 18.9 | - | 5.80 |
| 1 | 18.7 | 1.88 | 17.76 |
| 2 | 18.7 | 1.11 | 18.15 |
| 3 | 18.7 | 0.74 | 18.33 |

- Calculation

| CAPACITORS | ΔvL | VLDC |
| --- | --- | --- |
| 0 | - | 6.02 |
| 1 | 3.44 | 17.18 |
| 2 | 1.72 | 18.04 |
| 3 | 1.15 | 18.33 |

- Conclusion

Following the experiments & calculations, we can conclude that with the increase in capacitors, the output DC voltage also increases and the ripple decreases, smoothening the DC waveform.